CLEANING & MAINTAINING YOUR PREFINISHED TIMBER FLOOR

Prefinished floors are coated in a spray booth during the manufacturing process to ensure a perfect finish. Coating prior to being laid ensures a quicker, cleaner installation process. In general, sanding and reapplication of polyurethane coatings is necessary only when damage occurs or after 7-10 years of typical wear.

Polyurethane is not waterproof, only water-resistant and excess water will seep through the coatings causing damage. These floors do not need to be waxed, polished or buffed to maintain them – just cleaned.

IN GENERAL

Preferably use dry – not wet – cleaning methods for routine cleaning.

Keep dirt and grit off the floor as it can be ground in and damage coatings (using your vacuum cleaner with the brushes down or a dusting mop head on your Bona or Sabco mop).

Use protective mats inside and outside exterior doorways.

Wipe up spills ASAP with a slightly damp cloth and remove excess moisture with dry mop or cloth immediately.

Use mats and/or Slipstick protective floor felts under heavy furniture and movable objects (chairs, etc.) to prevent marking.

Don’t use excessive water or let water or wet items (such as towels) sit on timber flooring.

On completion of cleaning/mopping, the floor should be dry.

Don’t drag anything across the timber flooring, this can result in scratches, gouges & dents in the floor.

Don’t use steam mops, soaps, detergents, kerosene or “oil soaps”.  They may damage the wood and the finish and may leave a discoloured residue. The specialised cleaner specifically recommended for your floor coating is ideal.

EQUIPMENT, MATERIALS AND PERIODICITY

Vacuums and dry mops are both dry cleaning methods. Dry mops are not dipped in a bucket of water – water is sprayed on the floor or on to the mop head. We recommend a mop with a flat head and removable light microfiber cover/s.  Mops are available from Just Hardwood Floors.

Wet mopping is not recommended, but if wet mopping is necessary, a good quality mop which rolls/squeezes out almost all of the water is important. If using a wet mop a higher concentration of solution (2-4 times stronger) is required to stop water spots being left behind as the floor dries (cleaning solutions have a drying agent which stops water spots).

The suggested frequency for cleaning a timber floor is simple… as often as you would like. Vacuum or sweep the floor regularly to remove loose dirt, dust and gritty particles and prior to mopping OR, for light cleaning use a dry mop with an anti-static cover for picking up dust.  If you are using the correct cleaning method and solution, even daily cleaning will not damage your floor.  Most cleaning solutions can be diluted for general, heavy and spot cleaning.  The regularity of floor cleaning is often tied to the traffic level over the floor and the frequency of vacuuming.

PREFINISHED FLOORING

Bona Wood Floor Cleaner is a neutral floor cleaning solution designed for prefinished polyurethaned or prefinished oiled wood flooring.  Just spray and wipe clean.  It will effectively remove dirt and grime.  The 1 litre Spray is ready to use, no need to add water.  Final rinsing is not necessary.

Floor Cleaning Products

Bona Hard Floor Cleaner is the recommended product for cleaning pre-finished floors.

Unlike most other cleaning solutions, it will not leave a build-up or residue on the floor and it contains a drying

agent to prevent water marks being left after cleaning.

Bona Hard Floor Cleaner is available in a 1 litre “spray and wipe” bottle.

1 Litre Bona Spray Bottle$20.00 incl

Sabco Mop with Wet and Dry Covers$40.00 incl

Sabco Replacement Covers$15.00 incl

Bona Spray Mops$85.00 incl

Bona Replacement Covers$20.00 incl

Bona Concentrate 1 Litre$40.00 incl

Maintenance Products  (prices as at 1 April 2018)

The following products, available from our showroom and online, are recommended for use with prefinished flooring.  Visit us, our website or phone through your order and we can courier your order (delivery charge applies).

Slipstick Felt Pads

We highly recommend Slipstick self-adhesive pads to prevent your furniture damaging your timber floor. Slipstick pads are a heavy duty, dense, self-adhesive felt pad. Just peel off the backing and stick them to the bottom of your furniture. They come in a wide range of sizes that will suit anything you have in your home.

Felt pads should be placed on movable items (eg chairs) to prevent scratches and on non-movable items (eg tables and cabinets) to prevent dents. The felts will not compress with age and the self adhesive layer bonds firmly so they offer quality, durable protection.

Protects floors and walls		Adds feeling of softness

Superior quality			Makes furniture easier to move

Ultra-strength adhesive		Highly durable

19mm Round Discs Beige (24 Pieces)			$7.50 Incl.

25mm Round Discs Beige (16 Pieces) 		17042		$7.50 Incl.

38mm  Round Discs Beige (6 Pieces) 		17043		$8.50 incl.

114x152mm One Piece Rectangle Beige		17041		$12.50 incl.

Various Multipack Beige (33 Pieces)		23054		$13.50 incl.